QUADRAVERB 2  MIDI SYSEX SPECIFICATION

11/13/95

SYSTEM EXCLUSIVE FORMAT

The QuadraVerb 2  MIDI System Exclusive message format is as follows:

F0	System exclusive status

00 00 0E	Alesis manufacturer id#

0F	QuadraVerb id#

cc	Opcode

dd	Data

:	   :

:	   :

F7	End-Of-Exclusive

OPCODES:

01  -  MIDI Edit  F0 00 00 0E 0F 01 <function#> <block#> <page#>

<parameter#> <data> F7

<function#>  =   0= PARAMETER,  1= MIX,  2= GLOBAL, 3= TYPE*

<block#>   =   (1-8) # of block to be edited. Doesn’t matter for GLOBAL edits.

<page#>  =   (1-#of last page under function) page # for value to be edited.

<parameter#>  =   (1-#of last parameter on page) parameter # of value

<data> is in format described below.  It is a 16-bit value, which is transmitted in three MIDI bytes, LSB first.

TWO QUADRAVERB 2   BYTES:

0: A7 A6 A5 A4 A3 A2 A1 A0   (LSB)

1: B7 B6 B5 B4 B3 B2 B1 B0   (MSB)

TRANSMITTED AS:

0: 0 A7 A6 A5 A4 A3 A2 A1

1: 0 A0 B7 B6 B5 B4 B3 B2

2: 0 B1 B0 0  0  0  0  0

* Editing of a flashing TYPE parameter should be followed by an ‘ENTER’  MIDI sysex message (0B).

02  -  MIDI User Program Dump  F0 00 00 0E 0E 02 <prog.#> <data> F7

<program#>=  0..99 selects individual user bank 0 programs, 100 selects edit

<data> is in a packed format in order to optimize data transfer.  Eight MIDI bytes are used to transmit each block of 7 QuadraVerb 2  data bytes.  If the 7 data bytes are looked at as one 56-bit word, the format for transmission is eight 7-bit words beginning with the most significant bit of the first byte, as follows:

SEVEN QUADRAVERB 2   BYTES:

0: A7 A6 A5 A4 A3 A2 A1 A0

1: B7 B6 B5 B4 B3 B2 B1 B0

2: C7 C6 C5 C4 C3 C2 C1 C0

3: D7 D6 D5 D4 D3 D2 D1 D0

4: E7 E6 E5 E4 E3 E2 E1 E0

5: F7 F6 F5 F4 F3 F2 F1 F0

6: G7 G6 G5 G4 G3 G2 G1 G0

TRANSMITTED AS:

0: 0 A7 A6 A5 A4 A3 A2 A1

1: 0 A0 B7 B6 B5 B4 B3 B2

2: 0 B1 B0 C7 C6 C5 C4 C3

3: 0 C2 C1 C0 D7 D6 D5 D4

4: 0 D3 D2 D1 D0 E7 E6 E5

5: 0 E4 E3 E2 E1 E0 F7 F6

6: 0 F5 F4 F3 F2 F1 F0 G7

7: 0 G6 G5 G4 G3 G2 G1 G0

There are 306 data bytes sent for a single program dump, which corresponds to 256 bytes of program data.  With the header, the total number of bytes transmitted with a program dump is 314.  The location of each parameter within a program dump is shown in the next section: QuadraVerb 2   Program Data Format.

03 -  MIDI User Bank 0 Program Dump Request   F0 00 00 0E 0F 03 <program#> 	F7

<program#>=  0..99 selects individual user programs in User Bank 0, 100 selects

current edit, 101 is a dump all request

When received, the QuadraVerb 2  will respond to this message with a MIDI user program dump (02) of the User Bank 0 program or edit selected.

For the  dump all request User Bank 0 programs are dumped as individual program dumps (02) followed by all User Bank 1 programs dumped as individual program dumps (0C), a global parameter dump (07), and finally a MIDI program table dump (11).

07  -  MIDI Global Data Dump 	F0 00 00 0E 0F 07 <data> F7

<data> is in the same MIDI data format as described in 02, but with a different number of bytes due to the difference in the global parameter size.The global parameters for the MIDI dump are packed into 14 Quadraverb 2 bytes (see below) and transferred as 16 MIDI bytes.  The last byte packs the most significant bit of the footswitch head parameter into bit address 13:0 and the msb of the footswitch tail into bit address 13:1.   The following table outlines this:

GLOBAL PARAMETER PACKING:

Parameter name	    	lo lim	 hi lim	bits	 bit address

LCD contrast		1	10	8	0:7-0:0

Footswitch range head LSB*		0	299	8	1:7-1:0

Footswitch range tail LSB*		0	299	8	2:7-3:0

VU meter peak hold		0	1	8	3:7-3:0

Input audio source		0	1	8	4:7-4:0

Sample clock source		0	1	8	5:7-5:0

Digital left input channel		1	8	8	6:7-6:0

Digital right input channel		1	8	8	7:7-7:0

Digital left output channel		1	8	8	 8:7-8:0

Digital right output channel		1	8	8	9:7-9:0

Digital output mode		0	1	8	10:7-10:0

Tap tempo footswitch		0	2	8	11:7-11:0

Global direct signal		0	1	8	12:7-12:0

Footswitch range head MSB, tail MSB*	0	299	2	13:1-13:0

*The MSBs for the footswitch parameters are stored in bit address 13:1 and13:0.

Bits 13:7 through 13:2 should be set to 0.

08  -  MIDI Global Data Dump Request  F0 00 00 0E 0F 08 F7

When received, the QuadraVerb 2  will respond to this message with a MIDI global data dump (07).

09  -  MIDI Bypass or Compare  F0 00 00 0E 0F 09 <function> <off/on> F7

<function>  =   0= BYPASS, 1= COMPARE

<off/on>  =  0= function off, 1= function on

When received, the QuadraVerb 2  will respond to this message by turning BYPASS/COMPARE OFF/ON.

0A  -  MIDI DSP Error Message  F0 00 00 0E 0F 0A <data>  F7

<data>  =   1= “EFFECT MEMORY IS FULL”

2= “OUT OF LFOs”

4= “DSP IS FULL”

8= “BLOCK COMBINATION NOT ALLOWED”

0B  -  MIDI Enter Message  F0 00 00 0E 0F 0B F7

This performs the ENTER function on the current parameter.  It is useful for

acting on flashing parameters.

0C  -  MIDI User Bank 1 Program Dump  F0 00 00 0E 0E 0C<prog.#> <data> F7

<program#>=  0..99 selects individual User Bank 1 programs, 100 selects edit

<data> is the same format as the <data> of the User Bank 0 Dump opcode (02)

0D -   MIDI User Bank 1 Program Dump Request   F0 00 00 0E 0F 0D<program#>F7

<program#>=  0..99 selects individual user programs in User Bank 1, 100 selects

current edit, 101 is a dump all request

When received, the QuadraVerb 2  will respond to this message with a MIDI user program dump (0C) of the User Bank 1 program or edit selected.

For the  dump all request User Bank 0 programs are dumped as individual program dumps (02) followed by all User Bank 1 programs dumped as individual program dumps (0C), a global parameter dump (07), and finally a MIDI program table dump (11).

0F -   MIDI Block Bypass Dump     F0 00 00 0E 0F 0F <data> F7

<data> is in format described below.  It is an 8-bit value, which is transmitted in two MIDI bytes.  Bit A7 corresponds to the block bypass of block #8, likewise,

A6 corresponds to block #7 ... and A0 corresponds to block#1.  If thebit value= 1,

the block is bypassed; if thebit value= 0, the block is not bypassed.

ONE QUADRAVERB 2   BYTE:

0: A7 A6 A5 A4 A3 A2 A1 A0

TRANSMITTED AS:

0: 0 A7 A6 A5 A4 A3 A2 A1

1: 0 A0 0  0  0  0  0  0

10 -   MIDI Block Bypass Dump Request   F0 00 00 0E 0F 10 F7

When received, the QuadraVerb 2  will respond to this message with a MIDI Block Bypass Dump (0F).

11 -   MIDI Program Table Dump   F0 00 00 0E 0F 11 <data>  F7

<data> is in format described below.  The 128 MIDI program table entries are

9-bits each, and are transmitted in two MIDI bytes.  Values in the range 0-99 correspond to Presets 0-99, values 100-199 correspond to User bank 0 programs  0-99, and values 200-299 correspond to User bank 1 programs  0-99.

TWO QUADRAVERB 2   MIDI TABLE ENTRIES:

0: A7 A6 A5 A4 A3 A2 A1 A0

1:  0  0  0  0  0  0  0 A8

2: B7 B6 B5 B4 B3 B2 B1 B0

3:  0  0  0  0  0  0  0 B8

TRANSMITTED AS:

0: 0 A6 A5 A4 A3 A2 A1 A0

1: 0  0  0  0  0  0 A8 A7

2: 0 B6 B5 B4 B3 B2 B1 B0

3: 0  0  0  0  0  0 B8 B7

12 -   MIDI Program Table DumpRequest   F0 00 00 0E 0F 12 F7

When received, the QuadraVerb 2  will respond to this message with a MIDI Program Table Dump (11).

PARAMETER FORMAT

The following specific parameter information shows the locations in which each parameter resides after unpacking the data from its 7 bit MIDI format into the 8 bit format as described in opcode 02, program data dump.

GLOBAL DATA FORMAT

The Global parameters are comprised of 14 bytes of data.  Unlike the Program, the Global parameters are not packed.  This results in unused bits for most global bytes.  These bits must remain at 0. For direct parameter editing (sysex command 01H), the Page# and Parameter# are given in the table below.  The Function# should be set to 2 (=Global), and the Block# should be 1-8.

#	Parameter name	    	Page Parm	lo lim	 hi lim	bits	bit address

0.	LCD contrast		1	1	1	10	8	0:7-0:0

1.	Footswitch range head		2	1	0	299	16	2:7-1:0

2.	Footswitch range tail		2	2	0	299	16	4:7-3:0

3.	VU meter peak hold		3	1	0	1	8	5:7-5:0

4.	Input audio source		4	1	0	1	8	6:7-6:0

5.	Sample clock source		5	1	0	1	8	7:7-7:0

6.	Digital left input channel		6	2	1	8	8	8:7-8:0

7.	Digital right input channel		6	3	1	8	8	9:7-9:0

8.	Digital left output channel		6	4	1	8	8	10:7-10:0

9.	Digital right output channel		6	1	1	8	8	11:7-11:0

10.	Digital output mode		7	1	0	1	8	12:7-12:0

11.	Tap tempo footswitch		8	1	0	2	8	13:7-13:0

12.	Global direct signal		9	1	0	1	8	14:7-14:0

QUADRAVERB 2   PROGRAM DATA FORMAT

Programs contain 256 bytes of packed parameter data.  The first 92 bytes are common parameters for all programs.  The next 164 bytes depend on the number of bytes required for parameters of the effect blocks and the number of routes enabled.   Usually, not all of these bytes are used,  and in this case the end of the routing is designated by 0FFH, 0FFH.  Only numbers with negative offsets are in 2’s complement form.

#	Parameter name	 			lo lim	 hi lim	  bits byte:msb-byte:lsb

PROGRAM  NAME:

0.	Program name ASCII digit 0	 			32	127	8	0:7-0:0

...

13.	Program name ASCII digit 13	 			32	127	8	13:7-13:0

BLOCK TYPES:

14a.	Block 1 type definition	 			0	4 *	3	14:7-14:5

14b.	Block 1 subtype definition	 			0	     **	5	14:4-14:0

15a.	Block 2 type definition	 			0	4 *	3	15:7-15:5

...

20b.	Block 7 subtype definition	 			0	     **	5	20:4-20:0

21a.	Block 8 type definition	 			0	4 *	3	21:7-21:5

21b.	Block 8 subtype definition	 			0	     **	5	21:4-21:0

* Block type definitions are given in the next section:  Block type definitions

** The upper limit and definition for the block subtype depend on the block type.  The upper limit and definition for the subtype are given in the section:  Block subtype defintions

MODULATION:

22.	Mod. 1 source				0	129 *	8	22:7-22:0

23.	Mod. 1 target block #				1	11 **	8	23:7-23:0

24a.	Mod. 1 target type: parameter, route, or mix		0	2 ***	2	24:7-24:6

24b.	Mod. 1 target #				0	   ***	6	24:5-24:0

25.	Mod. 1 amplitude				-99	+99	8	25:7-25:0

26.	Mod. 2 source				0	129 *	8	26:7-26:0

...

49.	Mod. 7 amplitude				-99	+99	8	49:7-49:0

50.	Mod. 8 source				0	129 *	8	50:7-50:0

51.	Mod. 8 target block #				1	11 **	8	51:7-51:0

52a.	Mod. 8 target type: parameter, route, or mix		0	2 ***	2	52:7-52:6

52b.	Mod. 8 target #				0	   ***	6	52:5-52:0

53.	Mod. 8 amplitude				-99	+99	8	53:7-53:0

* Definitions of the modulation sources are in the section:  Modulation source definitions

** Target block # definition:  1 = block 1, ..., 8 = block 8, 9 = left out, 10 = right out, 11 = mix parameters.

*** Definitions of the modulation targets are in the section:  Modulation target definitions

BLOCK MIXES:

54.	Direct level	 			0	100	8	54:7-54:0

55.	Block 1 level	 			0	100	8	55:7-55:0

56.	Block 1 inmix level	 			0	100	8	56:7-56:0

57.	Block 2 level	 			0	100	8	57:7-57:0

...

68.	Block 7 inmix level	 			0	100	8	68:7-68:0

69.	Block 8 level	 			0	100	8	69:7-69:0

70.	Block 8 inmix level	 			0	100	8	70:7-70:0

71.	Master effects level	 			0	100	8	71:7-71:0

LOCAL GENERATORS:

72.	Generator 1 type	 			0	4 *	8	72:7-72:0

73.	Generator 1 trigger source channel	 			0	 2  **	8	73:7-73:0

74.	Generator 1 attack threshold	 			-32	0	8	74:7-74:0

75.	Generator 1 release threshold	 			-48	0	8	75:7-75:0

76.	Generator 1 attack time	 			0	250 ***	8	76:7-76:0

77.	Generator 1 release / decay time or period 	 	0	250 ***	8	77:7-77:0

78.	Generator 1 input gain	 			0	99	8	78:7-78:0

79.	Generator 1 envelope hold time	 			0	250 ***	8	79:7-79:0

80.	Generator 1 footswitch source		 		0	1 	8	80:7-80:0

81.	Generator 1 footswitch mode	 			0 	1	8	81:7-81:0

82.	Generator 2 type	 			0	4 *	8	82:7-82:0

83.	Generator 2 trigger source channel	 			0	 2  **	8	83:7-83:0

84.	Generator 2 attack threshold	 			-32	0	8	84:7-84:0

85.	Generator 2 release threshold	 			-48	0	8	85:7-85:0

86.	Generator 2 attack time	 			0	250 ***	8	86:7-86:0

87.	Generator 2 release / decay time or period 	 	0	250 ***	8	77:7-77:0

88.	Generator 2 input gain	 			0	99	8	88:7-88:0

89.	Generator 2 envelope hold time	 			0	250 ***	8	89:7-89:0

90.	Generator 2 footswitch source		 		0	1 	8	90:7-90:0

91.	Generator 2 footswitch mode	 			0 	1	8	91:7-91:0

* Generator type:  0 = Envelope generator,  1 = Peak follower, 2 = Ramp, 3 = LFO,                 4 = Footswitch

** Trigger source channel:  0 = Left channel,  1 = Right channel,  2 = Left & Right channels

*** This number is multiplied by 0.04 to get the decay in seconds.

BLOCK PARAMETERS

92.	Parameter #1 for 1st defined block*	 (MSB)		**	**	**	    **

93.	Parameter #1 for 1st defined block*	 (LSB)***	**	**	**	 **

94.	Parameter #2 for 1st defined block*	 (MSB)		**	**	**	    **

...

* Block parameters are not stored for blocks that are off. Parameters are stored in order of appearance on the Quadraverb 2  display - by block #, parameter page #, and from left to right within the page; the 1st stored parameter appears on the display at the left most block that is not off, on it’s 1st parameter page, at the left.

** The parameter definitions are given for each block type and subtype in the section: Block parameter definitions

*** Exists only if the block parameter requires two bytes.

BLOCK ROUTING

92*	Route #1 position byte (LSB)				0	255 **	8	92*:7-92*:0

93*	Route #1 position byte (MSB)				0	1     **	1	93*:7-93*:7

93*	Route #1 level 				0	58 ***	7	93*:6-93*:0

94*	Route #2 position byte (LSB)				0	255 **	8	94*:7-94*:0

...

* This number is offset by the number of block parameter bytes that are stored.

** An equation for calculating the route position byte is given in the section:  Route position definitions

*** This number has a corresponding dB level given by the table in the section:  Route level definitions

BLOCK BYPASS

255a.	Block Bypass for block #1 (1= Bypassed)			0	1	1	255:0-255:0

255b.Block Bypass for block #2 (1= Bypassed)			0	1	1	255:1-255:1

255c.	Block Bypass for block #3 (1= Bypassed)			0	1	1	255:2-255:2

255d.Block Bypass for block #4 (1= Bypassed)			0	1	1	255:3-255:3

255e.	Block Bypass for block #5 (1= Bypassed)			0	1	1	255:4-255:4

255f.	Block Bypass for block #6 (1= Bypassed)			0	1	1	255:5-255:5

255g.Block Bypass for block #7 (1= Bypassed)			0	1	1	255:6-255:6

255h.Block Bypass for block #8 (1= Bypassed)			0	1	1	255:7-255:7

TYPE DATA FORMAT

The number of Type parameters, and their location depend on whether the selectted block is defined or off .  The Type parameters are not packed.  This results in unused bits for most parameters.  These bits must remain at 0.  For direct parameter editing (sysex command 01H), the Page# and Parameter# are given in the table below.  The Function# should be set to 3 (=Type), and the Block# should be 1-8.

For  blocks that are off:

#	Parameter name	    	Page Parm	lo lim	 hi lim	bits	bit address

0.	Block type		1	1	0	4	8	0:7-0:0

1.	Block move source		2	1	1	8	8	1:7-1:0

2.	Block move destination		2	2	1	8	8	2:7-2:0

3.	Block copy / paste		3	1	0	1	8	3:7-3:0

For  blocks that are defined:

#	Parameter name	    	Page Parm	lo lim	 hi lim	bits	bit address

0.	Block type		1	1	0	4	8	0:7-0:0

1.	Block subtype		2	1	0	*	8	1:7-1:0

2.	Block move source		3	1	1	8	8	2:7-2:0

3.	Block move destination		3	2	1	8	8	3:7-3:0

4.	Block copy / paste		4	1	0	1	8	4:7-4:0

* The high limit depends on the block type, see the table Block subtype definitions.

BLOCK TYPE DEFINITIONS

This table defines the block types for a given parameter:

Parameter:

Block type:

0

OFF

1

EQ

2

PCH

3

DLY

4

RVB

BLOCK SUBTYPE DEFINITIONS

These tables define the block subtypes for a given parameter. Different block subtype tables are defined for  each block type.

Type = EQ

Parameter:

Block subtype

0

Lowpass filter

1

Bandpass filter

2

Highpass filter

3

Lowpass shelf eq

4

1 Band low parametric

5

1 Band high parametric

6

Highpass shelf eq

7

2 Band sweep shelf eq

8

3 Band parametric eq

9

4 Band parametric eq

10

5 Band graphic eq

11

Resonator

12

Mono tremolo

13

Stereo tremolo

14

Stereo simulator

15

Soft overdrive

16

Hard overdrive

17

Triggered Panning

18

Phase Inverter

Type = PCH

Parameter:

Block subtype

0

Mono chorus

1

Stereo chorus

2

Quad chorus

3

Mono flanging

4

Stereo flanging

5

Phasor

6

Mono lezlie

7

Stereo lezlie

8

Pitch shifter

9

Pitch detune

10

Ring modulator

11

Mono triggered flange

12

Stereo triggered flange

Type = DLY

Parameter:

Block subtype

0

Mono delay

1

Stereo delay

2

Ping pong delay

3

Multi tap delay

4

Tap tempo mono delay

5

Tap tempo ping pong delay

6

Sampling

Type = RVB

Parameter:

Block subtype

0

Mono room

1

Room      1

2

Hall         1

3

Plate        1

4

Chamber 1

5

Room      2

6

Hall         2

7

Plate        2

8

Chamber 2

9

Large plate

10

Large room

11

Spring

12

Nonlinear

13

Reverse

MODULATION SOURCE DEFINITIONS

Parameter:

Block subtype

0

Pitch bend

1

After touch

2

Note number

3

Note velocity

4

Controller 000

...

...

127

Controller 123

128

Local generator 1

129

Local generator 2

MODULATION TARGET DEFINITIONS

The modulation target type and target # make up an 8 bit word:  TTNNNNNN (where TT denotes the target type and NNNNNN denotes the target #).  The targets are for the effect block or mix specified by the target block #.

TT

Modulation target type

00

Block parameter

01

Route level

10

Mix level

If  TT = 00

NNNNNN

Modulation target parameter

0

Parameter #1

1

Parameter #2

...

...

32

Parameter #33

If  TT = 01

NNNNNN

Modulation target route

0

In left

1

In right

2

Block 1 inmix

3

Block 1 mix

4

Block 1 left

5

Block 1 right

6

Block 2 inmix

...

...

33

Block 8 right

If  TT = 10

NNNNNN

Modulation target mix

0

Direct level

1

Block 1 out level

2

Block 1 inmix level

3

Block 2 out level

4

Block 2 inmix level

...

...

16

Block 8 inmix level

17

Master fx level

BLOCK PARAMETER DEFINITIONS

These parameters are packed as 8 or 16 bit words MSB first in programs, but are always considered 16 bit words LSB first in parameter edits.

EQ:

Parameter

Page

Param

Bytes

Low limit

Hi limit

Step

LOWPASS FILTER:  type = 1, subtype = 0

lowpass fc

1

1

2

20

10,000

10

BANDPASS FILTER:  type = 1, subtype = 1

bandpass fc

1

1

2

20

4,000

10

bw

1

2

1

20

200

1

HIGHPASS FILTER:  type = 1, subtype = 2

highpass fc

1

1

2

200

20,000

10

LOWPASS SHELF EQ:  type = 001, subtype = 3

lowpass

1

1

2

20

10,000

10

gain

1

2

2

-140

+140

1

1 BAND LOW PARAMETRIC:  type = 1, subtype = 4

bandpass

1

1

2

20

2,000

10

gain

1

2

2

-140

+140

1

oct

1

3

1

20

200

1

1 BAND HIGH PARAMETRIC:  type = 1, subtype = 5

bp

1

1

2

1500

10,000

10

gain

1

2

2

-140

+140

1

oct

1

3

1

20

250

1

HIGHPASS SHELF EQ:  type = 1, subtype = 6

highpass

1

1

2

100

20,000

10

gain

1

2

2

-140

+140

1

2 BAND SWEEP SHELF EQ:  type = 1, subtype = 7

lowpass

1

1

2

20

10,000

10

gain

1

2

2

-140

+140

1

highpass

2

1

2

100

20,000

10

gain

2

2

2

-140

+140

1

3 BAND PARAMETRIC EQ:  type = 1, subtype = 8

lowpass

1

1

2

20

10,000

10

gain

1

2

2

-140

+140

1

mid band

2

1

2

20

2,000

1

dB

2

2

2

-140

+140

1

oct

2

3

1

20

200

1

highpass

3

1

2

100

20,000

10

gain

3

2

2

-140

+140

1

4 BAND PARAMETRIC EQ:  type = 1, subtype = 9

lowpass

1

1

2

20

10,000

10

gain

1

2

2

-140

+140

1

low mid

2

1

2

20

2,000

1

dB

2

2

2

-140

+140

1

oct

2

3

1

20

200

1

high mid

3

1

2

1500

10,000

10

dB

3

2

2

-140

+140

1

oct

3

3

1

20

250

1

highpass

4

1

2

100

20,000

10

gain

4

2

2

-140

+140

1

5 BAND GRAPHIC EQ:  type = 1, subtype = 10

63Hz

1

1

1

-14

+14

1

250Hz

1

2

1

-14

+14

1

1KHz

1

3

1

-14

+14

1

4KHz

1

4

1

-14

+14

1

16kHz

1

5

1

-14

+14

1

RESONATOR: type = 1, subtype = 11

tuning

1

1

1

0

60

1

decay

1

2

1

0

99

1

MONO TREMOLO:  type = 1, subtype = 12

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

shape

1

3

1

0

1

1

STEREO TREMOLO:  type = 1, subtype = 13

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

shape

1

3

1

0

1

1

STEREO SIMULATOR:  type = 1, subtype = 14

degrees

1

1

1

0

99

1

SOFT OVERDRIVE  type = 1, subtype =15

effect mix

1

1

1

0

99

1

brightness

2

1

1

0

99

1

dirve level

2

2

1

10

110

1

HARD OVERDRIVE  type = 1, subtype =16

effect mix

1

1

1

0

99

1

brightness

2

1

1

0

99

1

dirve level

2

2

1

10

110

1

TRIGGERED PANNING  type = 1, subtype =17

(modulation trig. 1 parm)

1

1

1

0

1

1

(modulation trig. 2 parm)

1

2

1

0

1

1

speed

1

3

1

0

99

1

doppler

1

4

1

0

99

1

distance

1

5

1

0

99

1

retrigger point

2

1

1

0

255

1

sweeping mode

3

1

1

0

1

1

trigger source

4

1

1

0

4

1

PHASE INVERTER  type = 1, subtype =18

output phase

1

1

1

0

4

1

PCH:

Parameter

Page

Param

Bytes

Low limit

Hi limit

Step

MONO CHORUS:  type = 2, subtype = 0

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

feedback

1

3

1

0

99

1

chorus predelay

2

1

1

0

100

1

STEREO CHORUS:  type = 2, subtype = 1

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

feedback

1

3

1

0

99

1

predelay chr1

2

1

1

0

100

1

chr2

2

2

1

0

100

1

QUAD CHORUS:  type = 2, subtype = 2

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

feedback

1

3

1

0

99

1

predelay chr1

2

1

1

0

100

1

chr2

2

2

1

0

100

1

predelay chr3

3

1

1

0

100

1

chr4

3

2

1

0

100

1

MONO FLANGING:  type = 2, subtype = 3

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

feedback

1

3

1

-99

99

1

flanging shape

2

1

1

0

1

1

STEREO FLANGING:  type = 2, subtype = 4

speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

feedback

1

3

1

-99

99

1

flanging shape

2

1

1

0

1

1

PHASOR:  type = 2, subtype = 5

phasor speed

1

1

1

0

99

1

depth

1

2

1

0

99

1

MONO LEZLIE:  type = 2, subtype = 6

lezlie motor

1

1

1

0

1

1

speed

1

2

1

0

1

1

high rotor level

2

1

1

-12

+6

1

STEREO LEZLIE:  type = 2, subtype = 7

lezlie motor

1

1

1

0

1

1

speed

1

2

1

0

1

1

high rotor level

2

1

1

-12

+6

1

stereo separation

3

1

1

-99

99

1

PITCH SHIFTER:  type = 2, subtype = 8

pitch shift fine

1

1

1

-99

+99

1

coarse

1

2

1

-12

+12

1

PITCH DETUNE:  type = 2, subtype = 9

detune amount

1

1

1

-99

+99

1

RING MODULATOR:  type = 2, subtype = 10

spectrum shift

1

1

1

-100

+100

1

TRIGGERED MONO FLANGING  type = 2, subtype =11

(modulation trig. 1 parm)

1

1

1

0

1

1

(modulation trig. 2 parm)

1

2

1

0

1

1

speed

1

3

1

0

99

1

depth

1

4

1

0

99

1

feedback

1

5

1

-99

99

1

retrigger point

2

1

1

0

255

1

sweeping mode

3

1

1

0

1

1

trigger source

4

1

1

0

4

1

TRIGGERED STEREO FLANGING  type = 2, subtype =12

(modulation trig. 1 parm)

1

1

1

0

1

1

(modulation trig. 2 parm)

1

2

1

0

1

1

speed

1

3

1

0

99

1

depth

1

4

1

0

99

1

feedback

1

5

1

-99

99

1

retrigger point

2

1

1

0

255

1

sweeping mode

3

1

1

0

1

1

trigger source

4

1

1

0

4

1

DLY:

Parameter

Page

Param

Bytes

Low limit

Hi limit

Step

MONO DELAY:  type = 3, subtype = 0

delay

1

1

2

1

50,000

1

note value

1

2

1

0

12

1

feedback

2

1

1

0

99

1

STEREO DELAY:  type = 3, subtype = 1

L dly

1

1

2

1

50,000

1

note value

1

2

1

0

12

1

L feedback

2

1

1

0

99

1

R dly

3

1

2

1

50,000

1

note value

3

2

1

0

12

1

R feedback

4

1

1

0

99

1

PING PONG DELAY:  type = 3, subtype = 2

delay

1

1

2

1

25,000

1

note value

1

2

1

0

12

1

feedback

2

1

1

0

99

1

MULTI TAP DELAY:  type = 3, subtype = 3

delay

1

1

2

1

50,000

1

tap level

1

2

1

0

38

1

L<- ->R

1

3

1

1

99

1

fb

1

4

1

0

99

1

delay

2

1

2

1

50,000

1

tap level

2

2

1

0

38

1

L<- ->R

2

3

1

1

99

1

fb

2

4

1

0

99

1

delay

3

1

2

1

50,000

1

tap level

3

2

1

0

38

1

L<- ->R

3

3

1

1

99

1

fb

3

4

1

0

99

1

delay

4

1

2

1

50,000

1

tap level

4

2

1

0

38

1

L<- ->R

4

3

1

1

99

1

fb

4

4

1

0

99

1

delay

5

1

2

1

50,000

1

tap level

5

2

1

0

38

1

L<- ->R

5

3

1

1

99

1

fb

5

4

1

0

99

1

master feedback

6

1

1

0

99

1

TAP TEMPO MONO DELAY:  type = 3, subtype = 4

delay

1

1

2

1

50,000

1

note value

1

2

1

0

38

1

feedback

2

1

1

0

99

1

TAP TEMPO PING PONG DELAY: type = 3, subtype = 5

delay

1

1

2

1

25,000

1

note value

1

2

1

0

38

1

feedback

2

1

1

0

99

1

SAMPLING: type = 3, subtype = 6

(modulation play trig. parm)

1

1

1

0

1

1

(modulation rec trig. parm)

1

2

1

0

1

1

play

1

3

1

0

2

1

rec

1

4

1

0

1

1

maximum sample length

2

1

2

2500

50,000

1

start

3

1

2

0

49,500

1

end

3

2

2

500

50,000

1

play trigger

4

1

1

0

6

1

rec trigger

5

1

1

0

6

1

midi note#

6

1

1

0

128

1

rec output

6

2

1

0

1

1

RVB:

Parameter

Page

Param

Bytes

Low limit

Hi limit

Step

MONO ROOM:  type = 4, subtype = 0

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gated level

5

2

1

0

99

1

gate hold

6

1

2

0

500

10

release

6

2

2

10

500

10

ROOM  1:  type = 4, subtype = 1

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gated level

7

2

1

0

99

1

gate hold

5

2

2

0

500

10

release

6

1

2

10

500

10

HALL  1:  type = 4, subtype = 2

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gated level

5

2

1

0

99

1

gate hold

6

1

2

0

500

10

release

6

2

2

10

500

10

PLATE  1:  type = 4, subtype = 3

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gated level

5

2

1

0

99

1

gate hold

6

1

2

0

500

10

release

6

2

2

10

500

10

CHAMBER  1:  type = 4, subtype = 4

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gated level

5

2

1

0

99

1

gate hold

6

1

2

0

500

10

release

6

2

2

10

500

10

ROOM  2:  type = 4, subtype = 5

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation swirl

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gated level

7

2

1

0

99

1

gate hold

8

1

2

0

500

10

release

8

2

2

10

500

10

HALL  2:  type = 4, subtype = 6

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation swirl

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gated level

7

2

1

0

99

1

gate hold

8

1

2

0

500

10

release

8

2

2

10

500

10

PLATE  2:  type = 4, subtype = 7

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation attack

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gated level

7

2

1

0

99

1

gated hold

8

1

2

0

500

10

release

8

2

2

10

500

10

CHAMBER  2:  type = 4, subtype = 8

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation attack

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gated level

7

2

1

0

99

1

gate hold

8

1

2

0

500

10

release

8

2

2

10

500

10

LARGE PLATE:  type = 4, subtype = 9

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation swirl

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gated level

7

2

1

0

99

1

gate hold

8

1

2

0

500

10

release

8

2

2

10

500

10

LARGE ROOM:  type = 4, subtype = 10

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

reflection level

5

1

1

0

99

1

spread

5

2

1

0

99

1

reverberation swirl

6

1

1

0

99

1

gate trig

7

1

1

0

2

1

gate level

7

2

1

0

99

1

gate hold

8

1

2

0

500

10

release

8

2

2

10

500

10

SPRING:  type = 4, subtype = 11

decay

1

1

1

0

99

1

damping hi

1

2

1

0

99

1

lo

1

3

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

gate trig

5

1

1

0

2

1

gate level

5

2

1

0

99

1

gate hold

6

1

2

0

500

10

release

6

2

2

10

500

10

NONLINEAR:  type = 4, subtype = 12

gate hold time

1

1

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

REVERSE:  type = 4, subtype = 13

reverse time

1

1

1

0

99

1

reverb density

2

1

1

0

99

1

diffusion

2

2

1

0

99

1

input high freq roll off

3

1

2

200

20,000

50

predelay

4

1

1

1

250

1

mix  direct<- ->dly

4

2

1

1

99

1

ROUTE POSITION DEFINITIONS

Route position byte (9bits)  = Sroute + [4*(Sblock-1)] +  [34*(Dblock-1)]

where,

Sroute= route source #:  0= Input L, 1= Input R, 2= Block inmix, 3= Block mix, 4= Block L, 5= Block R

Sblock= route source block #:  1= Input, 1= Block 1, ..., 8= Block 8

Dblock= route destination block #:  1= Block 1, ..., 8= Block 8, 9= Out L, 10= Out R

ROUTE LEVEL DEFINITIONS

Below is a table that gives the corresponding dB level for a given route level #:

Rout level #

Parameter dB

0

OFF

1

-48.0 dB

2

-42.0 dB

3

-38.0 dB

4

-36.0 dB

5

-34.0 dB

6

-32.0 dB

7

-31.0 dB

8

-30.0 dB

9

-29.0 dB

10

-28.0 dB

11

-27.0 dB

12

-26.0 dB

13

-25.0 dB

14

-24.0 dB

15

-23.0 dB

16

-22.0 dB

17

-21.0 dB

18

-20.0 dB

19

-19.5 dB

20

-19.0 dB

21

-18.5 dB

22

-18.0 dB

23

-17.5 dB

24

-17.0 dB

25

-16.5 dB

26

-16.0 dB

27

-15.5 dB

28

-15.0 dB

29

-14.5 dB

30

-14.0 dB

31

-13.5 dB

32

-13.0 dB

33

-12.5 dB

34

-12.0 dB

35

-11.5 dB

36

-11.0 dB

37

-10.5 dB

38

-10.0 dB

39

-9.5 dB

40

-9.0 dB

41

-8.5 dB

42

-8.0 dB

43

-7.5 dB

44

-7.0 dB

45

-6.5 dB

46

-6.0 dB

47

-5.5 dB

48

-5.0 dB

49

-4.5 dB

50

-4.0 dB

51

-3.5 dB

52

-3.0 dB

53

-2.5 dB

54

-2.0 dB

55

-1.5 dB

56

-1.0 dB

57

-0.5 dB

58

-0.0 dB