QUADRAVERB MIDI IMPLEMENTATION

SYSTEM EXCLUSIVE FORMAT

The QuadraVerb MIDI System Exclusive message format is as follows:

F0	System exclusive status

00 00 0E	Alesis manufacturer id#

02	Quadraverb id#

cc	Opcode

dd	Data

:	   :

:	   :

F7	End-Of-Exclusive

OPCODES:

01  -  MIDI Editting

F0 00 00 0E 02 01<function#><page#>< value1>< value2>< value3>F7

<function#> 	=   1=reverb, 2=delay, 3=pitch, 4=eq, 7=config, 8=mix, 9=mod, 10=name

<page#>       =  0..n       where n is the maximum page # for the selected function

<value1-3>   =  new parameter value in the following format:

Data:  B7 B6 B5 B4 B3 B2 B1 B0   (MSB)

A7 A6 A5 A4 A3 A2 A1 A0   (LSB)

Sent:   0   A7 A6 A5 A4 A3 A2 A1   <value1>

0   A0 B7 B6 B5 B4 B3 B2   <value2>

0   B1 B0  0    0    0    0   0    <value3>

All parameters to be edited must be sent in this format (12 MIDI bytes), regardless of the number of bits required to transmit the value of the parameter.  When the QuadraVerb receives this message, it will edit the specified parameter to the new value and display it.  If the function and page selected does not exist in the current configuration, the command will be ignored.  If the value received is out of range for the parameter selected, the range will be limited to a legal value.  The function and page numbers for each parameter are shown in the next section.

02  -  MIDI Data Dump 	F0 00 00 0E 02 02 <program#> <data> F7

<program#>	=  0..99 selects individual programs

=  100 selects the edit buffer

=  > 100  selects all 100 programs

<data> is in a packed format in order to optimize data transfer.  Eight MIDI bytes are used to transmit each block of 7 QuadraVerb data bytes.  If the 7 data bytes are looked at as one 56-bit word, the format for transmission is eight 7-bit words beginning with the most significant bit of the first byte, as follows:

SEVEN QUADRAVERB BYTES:

0: A7 A6 A5 A4 A3 A2 A1 A0

1: B7 B6 B5 B4 B3 B2 B1 B0

2: C7 C6 C5 C4 C3 C2 C1 C0

3: D7 D6 D5 D4 D3 D2 D1 D0

4: E7 E6 E5 E4 E3 E2 E1 E0

5: F7 F6 F5 F4 F3 F2 F1 F0

6: G7 G6 G5 G4 G3 G2 G1 G0

TRANSMITTED AS:

0: 0 A7 A6 A5 A4 A3 A2 A1

1: 0 A0 B7 B6 B5 B4 B3 B2

2: 0 B1 B0 C7 C6 C5 C4 C3

3: 0 C2 C1 C0 D7 D6 D5 D4

4: 0 D3 D2 D1 D0 E7 E6 E5

5: 0 E4 E3 E2 E1 E0 F7 F6

6: 0 F5 F4 F3 F2 F1 F0 G7

7: 0 G6 G5 G4 G3 G2 G1 G0

There are 147 bytes sent for a single data dump, which corresponds to 128 bytes of program data.  There are 14,629 bytes sent for a 100 program dump, which corresponds to 12,800 bytes of program data.  The location of each parameter within a program is shown in the next section.

When the QuadraVerb receives a data dump message, the display reads:

LOADING 
  MIDI DATA ...

Should the Quadraverb's MIDI input buffer overflow, the display will read:

MIDI INPUT
BUFFER OVERFLOW

This message will remain on the display until any button is pressed.

03  -  MIDI Dump Request

F0 00 00 0E 02 03 <program#> F7

<program#>	=  0..99 selects individual programs

=  100 selects the edit buffer

=  > 100  selects all 100 programs

When this message is received, a MIDI data dump will be initiated, and the display will read:

MIDI DATA DUMP
  IN PROGRESS

Upon completion of the dump, the display will return to its previously displayed page.

INDIVIDUAL PARAMETER LOCATION FUNCTION AND PAGE VALUES

The following lists give the parameter locations and ranges for each of the effects in each of the configurations.  The parameter associated with a particular function and page may be dependent on both the current configuration, and the current mode of the effect (e.g., chorus, phase, etc.).  In conditions where a page relates to different parameters depending on the mode of the effect, the parameters are listed in tables under each mode.  All parameters are offset binary values.  This means that a signed parameter with a range of -99 to +99 in the display is actually stored as 0 for -99, 99 for 0, and 198 for +99.

REVERB (FUNCTION 1)

CONFIGURATION 0:

page

0 (type)	0 (PLATE 1)	1 (ROOM 1)	2 (CHAMBER 1)	3 (HALL 1)	4 (REVERSE 1)

1	0-3 (INPUT 1)	0-3 (INPUT 1)	0-3 (INPUT 1)	0-3 (INPUT 1)	0-3 (INPUT 1)

2	0-1 (INPUT 2)	0-1 (INPUT 2)	0-1 (INPUT 2)	0-1 (INPUT 2)	0-1 (INPUT 2)

3	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)

4	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)

5	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)

6	0-99 (DECAY)	0-99 (DECAY)	0-99 (DECAY)	0-99 (DECAY)	0-99 (REV TIME)

7	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)

8	0-8 (DENSITY)	0-8 (DENSITY)	0-8 (DENSITY)	0-60 (LOW DEC)	0-8 (DENSITY)

9	0-60 (LOW DEC)	0-60 (LOW DEC)	0-60 (LOW DEC)	0-60 (HIGH DEC)	0-60 (LOW DEC)

10	0-60 (HIGH DEC)	0-60 (HIGH DEC)	0-60 (HIGH DEC)	0-1 (GATE ON)	0-60 (HIGH DEC)

11	0-1 (GATE ON)	0-1 (GATE ON)	0-1 (GATE ON)	0-99 (GATE HLD)

12	0-99 (GATE HLD)	0-99 (GATE HLD)	0-99 (GATE HLD)	0-99 (GATE REL)

13	0-99 (GATE REL)	0-99 (GATE REL)	0-99 (GATE REL)	0-99 (GATE LEV)

14	0-99 (GATE LEV)	0-99 (GATE LEV)	0-99 (GATE LEV)

CONFIGURATION 1:

page

0 (type)	0 (PLATE 1)	1 (ROOM 1)	2 (CHAMBER 1)	3 (HALL 1)	4 (REVERSE 1)

1	0-2 (INPUT 1)	0-2 (INPUT 1)	0-2 (INPUT 1)	0-2 (INPUT 1)	0-2 (INPUT 1)

2-14 same as configuration 0

CONFIGURATION 2 & 3:

Not used

CONFIGURATION 4:

page

0 (type)	0 (PLATE 2)	1 (ROOM 2)	2 (CHAMBER 2)	3 (HALL 2)	4 (REVERSE 2)

1	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)

2	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)	1-140 (PREDLY)

3	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)	1-198 (PRE MIX)

4	0-99 (DECAY)	0-99 (DECAY)	0-99 (DECAY)	0-99 (DECAY)	0-99 (REV TIME)

5	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)	0-8 (DIFFUSION)

6	0-8 (DENSITY)	0-8 (DENSITY)	0-8 (DENSITY)	0-60 (LOW DEC)	0-8 (DENSITY)

7	0-60 (LOW DEC)	0-60 (LOW DEC)	0-60 (LOW DEC)	0-60 (HIGH DEC)	0-60 (LOW DEC)

8	0-60 (HIGH DEC)	0-60 (HIGH DEC)	0-60 (HIGH DEC)	0-1 (GATE ON)	0-60 (HIGH DEC)

9	0-1 (GATE ON)	0-1 (GATE ON)	0-1 (GATE ON)	0-99 (GATE HLD)

10	0-99 (GATE HLD)	0-99 (GATE HLD)	0-99 (GATE HLD)	0-99 (GATE REL)

11	0-99 (GATE REL)	0-99 (GATE REL)	0-99 (GATE REL)	0-99 (GATE LEV)

12	0-99 (GATE LEV)	0-99 (GATE LEV)	0-99 (GATE LEV)

DELAY (FUNCTION 2)

CONFIGURATION 0:

page

0 (type)	0 (MONO)	1 (STEREO)	2 (PING-PONG)
1	0-1 (INPUT 1)	0-1 (INPUT 1)	0-1 (INPUT 1)
2	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)
3	1-800 (DELAY)	1-400 (L DELAY)	1-400 (DELAY)
4	0-99 (FEEDB)	0-99 (L FEEDB)	0-99 (FEEDB)
5		1-400 (R DELAY)
6		0-99 (R FEEDB)

CONFIGURATION 1:

page

0 (type)	0 (MONO)	1 (STEREO)	2 (PING-PONG)

1	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)

2	1-800 (DELAY)	1-400 (L DELAY)	1-400 (DELAY)

3	0-99 (FEEDB)	0-99 (L FEEDB)	0-99 (FEEDB)

4		1-400 (R DELAY)

5		0-99 (R FEEDB)

CONFIGURATION 2:

page

0 (type)	0 (MONO)	1 (STEREO)	2 (PING-PONG)

1	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)

2	1-1500 (DELAY)	1-750 (L DELAY)	1-750 (DELAY)

3	0-99 (FEEDB)	0-99 (L FEEDB)	0-99 (FEEDB)

4		1-750 (R DELAY)

5		0-99 (R FEEDB)

CONFIGURATION 3:

page

0 (type)	0 (MONO)	1 (STEREO)	2 (PING-PONG)

1	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)

2	0-198 (IN MIX)	0-198 (IN MIX)	0-198 (IN MIX)

3	1-1500 (DELAY)	1-750 (L DELAY)	1-750 (DELAY)

4	0-99 (FEEDB)	0-99 (L FEEDB)	0-99 (FEEDB)

5		1-750 (R DELAY)

6		0-99 (R FEEDB)

CONFIGURATION 4:

Not used

PITCH (FUNCTION 3)

CONFIGURATION 0 & 3:
page
0 (type)	0 (M CHORUS)	1 (S CHORUS)	2 (M FLANGE)	3 (S FLANGE)	4 (DETUNE)	5 (PHASER)
1	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)	0-1 (INPUT)
2	0-1 (WAVE)	0-1 (WAVE)	0-99 (SPEED)	0-99 (SPEED)	0-198 (TUNE)	0-99 (SPEED)
3	0-99 (SPEED)	0-99 (SPEED)	0-99 (DEPTH)	0-99 (DEPTH)		0-99 (DEPTH)
4	0-99 (DEPTH)	0-99 (DEPTH)	0-99 (FBACK)	0-99 (FBACK)
5	0-99 (FBACK)	0-99 (FBACK)	0-1 (TRIGGER)	0-1 (TRIGGER)

CONFIGURATION 1:

page

0	0-99 (SEPARATION)

1	0-1 (MOTOR)

2	0-1 (SPEED)

CONFIGURATION 2:

Not used

CONFIGURATION 4:

page

0 (type)	0 (CHORUS OFF)	1 (CHORUS ON)

1		0-1 (WAVE)

2		0-99 (SPEED)

3		0-99 (DEPTH)

EQ (FUNCTION 4)

CONFIGURATION 0 & 4:

page

0	20-999 (LOW FREQUENCY)

1	0-560 (LOW BOOST/CUT)

2	200-9999 (MID FREQUENCY)

3	20-255 (MID BANDWIDTH)

4	0-560 (MID BOOST/CUT)

5	2000-18000 (HIGH FREQUENCY)

6	0-560 (HIGH BOOST/CUT)

CONFIGURATION 1:

page

0	0-26 (HIGH ROTOR LEVEL)

CONFIGURATION 2:

page

0	0-28 (16Hz)

1	0-28 (32Hz)

2	0-28 (62Hz)

3	0-28 (126Hz)

4	0-28 (250Hz)

5	0-28 (500Hz)

6	0-28 (1KHz)

7	0-28 (2KHz)

8	0-28 (4KHz)

9	0-28 (8KHz)

10	0-28 (16KHz)

CONFIGURATION 3:

page

0	20-999 (LOW FREQUENCY)

1	0-560 (LOW BOOST/CUT)

2	20-500 (LOW MID FREQUENCY)

3	20-255 (LOW MID BANDWIDTH)

4	0-560 (LOW MID BOOST/CUT)

5	200-9999 (MID FREQUENCY)

6	20-255 (MID BANDWIDTH)

7	0-560 (MID BOOST/CUT)

8	2000-18000 (HIGH MID FREQUENCY)

9	20-255 (HIGH MID BANDWIDTH)

10	0-560 (HIGH MID BOOST/CUT)

11	2000-18000 (HIGH FREQUENCY)

12	0-560 (HIGH BOOST/CUT)

CONFIGURATION (FUNCTION 7)

page

0	0-4 (CONFIGURATION)

0:  EQ>PCH>DL>REVERB

1:  LESLIE>DL>REVERB

2:  GRAPHIC EQ>DELAY

3:  5BAND EQ>PCH>DLY

4:  3 BAND EQ>REVERB

MIX (FUNCTION 8)

CONFIGURATION 0:

page

0 (type)	0 (PRE-EQ)	1 (POST-EQ)

1	0-99 (DIRECT)	0-99 (MAST FX)

2	0-99 (MAST FX)	0-99 (EQ)

3	0-99 (PITCH)	0-99 (PITCH)

4	0-99 (DELAY)	0-99 (DELAY)

5	0-99 (REVERB)	0-99 (REVERB)

CONFIGURATION 1:

page

0	0-99 (MAST FX)

1	0-99 (LESLIE)

2	0-99 (DELAY)

3	0-99 (REVERB)

CONFIGURATION 2:

page

0	0-99 (MAST FX)

1	0-99 (EQ)

2	0-99 (DELAY)

CONFIGURATION 3:
page
0 (type)	0 (PRE-EQ)	1 (POST-EQ)
1	0-99 (DIRECT)	0-99 (MAST FX)
2	0-99 (MAST FX)	0-99 (EQ)
3	0-99 (PITCH)	0-99 (PITCH)
4	0-99 (DELAY)	0-99 (DELAY)

CONFIGURATION 4:

page

0 (type)	0 (PRE-EQ)	1 (POST-EQ)

1	0-99 (DIRECT)	0-99 (MAST FX)

2	0-99 (MAST FX)	0-99 (EQ)

5	0-99 (REVERB)	0-99 (REVERB)

MOD (FUNCTION 9)

CONFIGURATION 0-4:

page

0	0-126 (MOD 1 SOURCE)

1	0-? (MOD 1 TARGET)

2	0-198 (MOD 1 AMPLITUDE)

3	0-126 (MOD 2 SOURCE)

4	0-? (MOD 2 TARGET)

5	0-198 (MOD 2 AMPLITUDE)

6	0-126 (MOD 3 SOURCE)

7	0-? (MOD 3 TARGET)

8	0-198 (MOD 3 AMPLITUDE)

9	0-126 (MOD 4 SOURCE)

10	0-? (MOD 4 TARGET)

11	0-198 (MOD 4 AMPLITUDE)

12	0-126 (MOD 5 SOURCE)

13	0-? (MOD 5 TARGET)

14	0-198 (MOD 5 AMPLITUDE)

15	0-126 (MOD 6 SOURCE)

16	0-? (MOD 6 TARGET)

17	0-198 (MOD 6 AMPLITUDE)

18	0-126 (MOD 7 SOURCE)

19	0-? (MOD 7 TARGET)

20	0-198 (MOD 7 AMPLITUDE)

21	0-126 (MOD 8 SOURCE)

22	0-? (MOD 8 TARGET)

23	0-198 (MOD 8 AMPLITUDE)

Modulation target values are dependent on the current configuration and mode of a program.  The most significant nibble determines the function being modulated as follows:

0XH:	REVERB MOD

1XH:	DELAY MOD

2XH:	PITCH MOD

3XH:	EQ MOD

4XH:	MIX MOD

The possible modulation targets within each of the above is as follows:

REVERB CONFIGURATION 0,1,4 (nothing in 2 & 3):

Target	PLATE	ROOM	CHAMBER	HALL	REVERSE	
00H	IN MIX	IN MIX	IN MIX	IN MIX	IN MIX
01H	PREDLY	PREDLY	PREDLY	PREDLY	PREDLY
02H	PRE MIX	PRE MIX	PRE MIX	PRE MIX	PRE MIX
03H	DECAY	DECAY	DECAY	0DECAY	REV TIME
04H	DIFFUSION	DIFFUSION	DIFFUSION	DIFFUSION	DIFFUSION
05H	DENSITY	DENSITY	DENSITY	LOW DEC	DENSITY
06H	LOW DEC	LOW DEC	LOW DEC	HIGH DEC	LOW DEC
07H	HIGH DEC	HIGH DEC	HIGH DEC		HIGH DEC

DELAY CONFIGURATION 0,1,3 (nothing in 4):

Target	MONO	STEREO	PING-PONG

10H	IN MIX	IN MIX	IN MIX
11H	DELAY	L DELAY	DELAY
12H	FEEDB	L FEEDB	FEEDB

13H		R DELAY

14H		R FEEDB

CONFIGURATION 2:

Target	MONO	STEREO	PING-PONG

10H	DELAY	L DELAY	DELAY
11H	FEEDB	L FEEDB	FEEDB

12H		R DELAY

13H		R FEEDB

PITCH CONFIGURATION 0 & 3 (nothing in 2):

Target	M CHORUS	S CHORUS	M FLANGE	S FLANGE	DETUNE	PHASER

20H	SPEED	SPEED	SPEED	SPEED	DETUNE	SPEED

21H	DEPTH	DEPTH	DEPTH	DEPTH		DEPTH

22H	FBACK	FBACK	FBACK	FBACK

CONFIGURATION 1:

Target

20H	SEPARATION

21H	MOTOR

22H	SPEED

CONFIGURATION 4:

Target

20H	SPEED

21H	DEPTH

EQ CONFIGURATION 0 & 4:

Target

30H	LOW FREQUENCY

31H	LOW BOOST/CUT

32H	MID FREQUENCY

33H	MID BANDWIDTH

34H	MID BOOST/CUT

35H	HIGH FREQUENCY

36H	HIGH BOOST/CUT

CONFIGURATION 1:

Target

30H	HIGH ROTOR LEVEL

CONFIGURATION 2:

Target

30H	16Hz

31H	32Hz

32H	62Hz

33H	126Hz

34H	250Hz

35H	500Hz

36H	1KHz

37H	2KHz

38H	4KHz

39H	8KHz

3AH	16KHz

CONFIGURATION 3:

Target

30H	LOW FREQUENCY

31H	LOW BOOST/CUT

32H	LOW MID FREQUENCY

33H	LOW MID BANDWIDTH

34H	LOW MID BOOST/CUT

35H	MID FREQUENCY

36H	MID BANDWIDTH

37H	MID BOOST/CUT

38H	HIGH MID FREQUENCY

39H	HIGH MID BANDWIDTH

3AH	HIGH MID BOOST/CUT

3BH	HIGH FREQUENCY

3CH	HIGH BOOST/CUT

MIX CONFIGURATION 0:

Target	PRE-EQ	POST-EQ

40H	DIRECT	MAST FX

41H	MAST FX	EQ

42H	PITCH	PITCH

43H	DELAY	DELAY

44H	REVERB	REVERB

CONFIGURATION 1:

Target

40H	MAST FX

41H	LESLIE

42H	DELAY

43H	REVERB

CONFIGURATION 2:

Target

40H	MAST FX

41H	EQ

42H	DELAY

CONFIGURATION 3:
Target
40H	DIRECT
41H	MAST FX
42H	PITCH
43H	DELAY

CONFIGURATION 4:

Target

40H	DIRECT

41H	MAST FX

42H	REVERB

NAME (FUNCTION 10)

CONFIGURATION 0-4:

page

0	32-127 (1st DIGIT NAME)

1	32-127 (2nd DIGIT NAME)

2	32-127 (3rd DIGIT NAME)

3	32-127 (4th DIGIT NAME)

4	32-127 (5th DIGIT NAME)

5	32-127 (6th DIGIT NAME)

6	32-127 (7th DIGIT NAME)

7	32-127 (8th DIGIT NAME)

8	32-127 (9th DIGIT NAME)

9	32-127 (10th DIGIT NAME)

10	32-127 (11th DIGIT NAME)

11	32-127 (12th DIGIT NAME)

12	32-127 (13th DIGIT NAME)

13	32-127 (14th DIGIT NAME)

PARAMETER ADDRESSES WITHIN PROGRAMS

The following addresses (next page) are relative to the first address of a program.  These addresses assume that the data has been unpacked into byte values.  Blank locations should be left 0.

| 00 LOW EQ FREQ MSB 01 LOW EQ FREQ LSB / 16Hz 02 LOW EQ AMP MSB / 32Hz 03 LOW EQ AMP LSB / 62Hz 04 MID EQ FREQ MSB / 126Hz 05 MID EQ FREQ LSB / 250Hz 06 MID EQ BANDWIDTH / 500Hz 07 MID EQ AMP MSB / 1KHz 08 MID EQ AMP LSB / 2KHz 09 HIGH EQ FREQ MSB / 4KHz 0A HIGH EQ FREQ LSB / 8KHz 0B HIGH EQ AMP MSB / 16KHz 0C HIGH EQ AMP LSB 0D LESLIE HIGH ROTOR LEVEL 0E LOW MID EQ FREQ MSB 0F LOW MID EQ FREQ LSB 10 LOW MID EQ BANDWIDTH 11 LOW MID EQ AMP MSB 12 LOW MID EQ AMP LSB 13 HIGH MID EQ FREQ MSB 14 HIGH MID EQ FREQ LSB 15 HIGH MID EQ BANDWIDTH 16 HIGH MID EQ AMP MSB 17 HIGH MID EQ AMP LSB 18-19 1A PITCH MODE 1B PITCH INPUT 1C LFO WAVEFORM 1D LFO SPEED 1E LFO DEPTH 1F 20 PITCH FEEDBACK 21 DETUNE AMOUNT 22 LESLIE SEPARATION 23 LESLIE MOTOR 24 LESLIE SPEED 25 TRIGGER FLANGE 26 27 DELAY MODE 28 DELAY INPUT 29 DELAY INPUT MIX 2A DELAY MSB / LEFT DELAY MSB 2B DELAY LSB / LEFT DELAY LSB 2C FEEDBACK / LEFT FEEDBACK 2D RIGHT DELAY MSB 2E RIGHT DELAY LSB 2F RIGHT FEEDBACK 30-31 32 REVERB MODE 33 34 REVERB INPUT 1 35 REVERB INPUT 2 36 REVERB INPUT MIX 37 REVERB PREDELAY 38 REVERB PREDELAY MIX 39 REVERB DECAY 3A REVERB DIFFUSION | 3B REVERB LOW DECAY 3C REVERB HIGH DECAY 3D REVERB DENSITY 3E REVERB GATE 3F REVERB GATE HOLD 40 REVERB GATE RELEASE 41 REVERB GATED LEVEL 42-43 44 CONFIGURATION 45 MIX DIRECT PRE/POST 46 DIRECT LEVEL / EQ LEVEL 47 MASTER EFFECTS LEVEL 48 PITCH LEVEL / LESLIE LEVEL 49 DELAY LEVEL 4A REVERB LEVEL 4B-4F 50 MOD 1 SOURCE 51 MOD 1 TARGET 52 MOD 1 AMPLITUDE 53 MOD 2 SOURCE 54 MOD 2 TARGET 55 MOD 2 AMPLITUDE 56 MOD 3 SOURCE 57 MOD 3 TARGET 58 MOD 3 AMPLITUDE 59 MOD 4 SOURCE 5A MOD 4 TARGET 5B MOD 4 AMPLITUDE 5C MOD 5 SOURCE 5D MOD 5 TARGET 5E MOD 5 AMPLITUDE 5F MOD 6 SOURCE 60 MOD 6 TARGET 61 MOD 6 AMPLITUDE 62 MOD 7 SOURCE 63 MOD 7 TARGET 64 MOD 7 AMPLITUDE 65 MOD 8 SOURCE 66 MOD 8 TARGET 67 MOD 8 AMPLITUDE 68-69 6A 1ST DIGIT NAME 6B 2ND DIGIT NAME 6C 3RD DIGIT NAME 6D 4TH DIGIT NAME 6E 5TH DIGIT NAME 6F 6TH DIGIT NAME 70 7TH DIGIT NAME 71 8TH DIGIT NAME 72 9TH DIGIT NAME 73 10TH DIGIT NAME 74 11TH DIGIT NAME 75 12TH DIGIT NAME 76 13TH DIGIT NAME 77 14TH DIGIT NAME 78-7F |
| --- | --- |